301.013 ‘ how to’

no jtag port  with single chip = 4MB dump

Using a solder type jtag  [with 100K resistors]

1] remove board from IRD housing and flip board over to access 18 pin jtag

2] < mother board rear >

- 2   3   4   5   6   7   8   9

==  PADS

- 11 12  13 14 15 16 17 18

3] Solder  jtag      # 2  to pad 2

3 to 3

4 to 4

5 to 6

13 to 5

18 thru 25  to 10

4] Plug parallel cable in to printer port and plug power to IRD

5] Use jkeys 2.01  and you will see  F’s as box keys

a] choose “ ird model” as [unknown]

b]  “Region” is [ user specific]

c] Start = 7FC00000

d] Bytes = 400000

e] save memory    [ DO NOT ERASE OR REFLASH!! ]

6] read 4 MB dump in ‘Ultra edit” at address

a]  003FFF40  =  ird

b] 003FFF44   =Boxkeys

[ This was my way of retreiving info!  Use at your own risk! ]

There are other ways to do this. All new info would be appreciated 

Included are some other files and read  me’s

Thanks goes to the guys on several forums who helped on my info. Especially Dave2 for jkeys.