// jKeys IRD and Flash Definitions

//  Original entries by Dave2

//

// Release Notes (chronological starting with newest)

//   dd-Mmm-YYYY Subsequent release notes (author)

//   03-Feb-2003 Initial Release with jKeys 2.9.9 (Dave2)

//

Release, 1,0,0

// ============================================================================

// ============================================================================

//Microprocessor JTAG Identification Codes

Micro, 1,  1, "STi5500",  0xd4c9041, 0xfffffff

Micro, 2,  1, "ST20-TP2", 0x5193041, 0xfffffff

Micro, 3,  1, "ST20-TP4", 0x5198041, 0xfffffff

Micro, 4,  1, "ST20-GP6", 0x5196041, 0xfffffff

Micro, 5,  1, "STi5505",  0xd4cb041, 0xfffffff

Micro, 6,  1, "STi5508",  0xd402041, 0xfffffff

Micro, 7,  1, "STi5518",  0xd502041, 0xfffffff

Micro, 8,  1, "STi5518MVB-X", 0xd405041, 0xfffffff

Micro, 9,  1, "STi5510",  0xd4cd041, 0xfffffff

Micro, 10, 1,  "Unknown ST device", 0x041,0xfff

Micro, 11, 2, "LSI SC2000",0x400006d,0xfffffff

Micro, 12, 2, "Unknown LSI device", 0x06d, 0xfff

// ============================================================================

// ============================================================================

// ============================================================================

// ============================================================================

// IRD Defintions/IRD Flash Defintions

IRD, 1, "2700", 1, 1, 1, 1, 2, 2, 0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFEFFC4, 0x7FFEFFC8, "10G", 1, 1

IRDFlash, 1, "Flash 1(29F400)", 0x2223, 0x7FF80000, 0x80000, 2, 2, 0

IRD, 2, "2800", 1, 1, 1, 1, 2, 2, 0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFEFFC4, 0x7FFEFFC8, "11R", 1, 1

IRDFlash, 2, "Flash 1(29F400)", 0x2223, 0x7FF80000, 0x80000, 2, 2, 0

IRD, 3, "3700", 1, 1, 1, 1, 2, 2, 0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFEFFC4, 0x7FFEFFC8, "10G", 1, 1

IRDFlash, 3, "Flash 1(29F400)", 0x2223, 0x7FF80000, 0x80000, 2, 2, 0

IRD, 4, "3800", 1, 1, 1, 1, 2, 2, 0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFEFFC4, 0x7FFEFFC8, "10L", 1, 1

IRDFlash, 4, "Flash 1(29LV160)", 0x22C4, 0x7FE00000, 0x200000, 2, 2, 0

IRD, 5, "3900", 1, 1, 1, 1, 2, 2, 0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFEFFF4, 0x7FFEFFF8, "11L", 1, 1

IRDFlash, 5, "Flash 1(29LV160)", 0x22C4, 0x7FE00000, 0x200000, 2, 2, 0

IRD, 6, "4700", 1, 1, 1, 1, 2, 2, 0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFEFFC4, 0x7FFEFFC8, "10H", 1, 1

IRDFlash, 6, "Flash 1(29LV160)", 0x22C4, 0x7FE00000, 0x200000, 2, 2, 0

IRD, 7, "4900", 1, 1, 1, 1, 2, 2, 0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFEFFF4, 0x7FFEFFF8, "11H", 1, 1

IRDFlash, 7, "Flash 1(29LV160)", 0x22C4, 0x7FE00000, 0x200000, 2, 2, 0

IRD, 8, "501-5100", 6, 3, 1, 1, 2, 2, 0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFDFFF4, 0x7FFDFFF8, "10S", 3, 4

IRDFlash, 8, "Flash 1(U6-29DL323)", 0x2250, 0x7F800000, 0x400000, 2, 2, 0

IRDFlash, 8, "Flash 2(U7-29DL323)", 0x2250, 0x7FC00000, 0x400000, 2, 2, 0

IRD, 9, "301-3100", 7, 3, 1, 1, 2, 2,	0x7FFFFFC0, 0x7FFFFFC4, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFDFFF4, 0x7FFDFFF8, "10E", 3, 4

IRDFlash, 9, "Flash 1(U10-M29W160)", 0x22C4, 0x7FC00000, 0x200000, 2, 2, 0

IRDFlash, 9, "Flash 2(U12-M29W160)", 0x22C4, 0x7FE00000, 0x200000, 2, 2, 0

IRD, 10, "6000", 3, 4, 1, 1, 2, 4, 0x7FFFFFC0, 0x7FFFFFC8, 0x7FFFFFA0, 0x7FFFFFA8, 0x7FFDFFF4, 0x7FFDFFF8, "12N", 4, 2

IRDFlash, 10, "Flash 1 Hi(U45-29LV160)", 0x22C4, 0x7FC00000, 0x200000, 2, 4, 2

IRDFlash, 10, "Flash 2 Lo(U41-29LV160)", 0x22C4, 0x7FC00000, 0x200000, 2, 4, 0

IRD, 11, "301-3100", 11, 2, 1, 2, 2, 2, 0x1FC0FFF0, 0x1FC0FFF4, 0x1FC0FFC0, 0x1FC0FFC8, 0x1FFEFFF4, 0x1FFEFFF8, "10U", 1, 3

IRDFlash, 11, "Flash 1(IC22-29LV160)", 0x2250, 0x1FE00000, 0x200000, 2, 2, 0

IRDFlash, 11, "Flash 2(IC23-29LV160)", 0x2250, 0x1FC00000, 0x200000, 2, 2, 0

IRD, 12, "DRD420RE", 1, 1, 2, 1, 2, 2, 0, 0, 0, 0, 0, 0x5000003D , "XXX", 3, 1

IRDFlash, 12, "Flash 1(28F160)", 0xd0, 0x50000000, 0x200000, 2, 2, 0

IRDFlash, 12, "Flash 2(MX29F800)", 0x0, 0x7FF00000, 0x100000, 2, 2, 0

IRD, 13, "DRD430", 7, 1, 2, 1, 2, 2, 0, 0, 0, 0, 0, 0x5000003D , "XXX", 5, 4

IRDFlash, 13, "Flash 1(M29W320DB)", 0x16, 0x7FC00000, 0x400000, 2, 2, 0

IRD, 14, "DRD431RG", 7, 1, 2, 1, 2, 2, 0, 0, 0, 0, 0, 0x5000003D , "XXX", 5, 4

IRDFlash, 14, "Flash 1(28F320J3A)", 0x16, 0x7FC00000, 0x400000, 2, 2, 0

IRD, 15, "DRD220", 1, 1, 2, 1, 2, 2, 0, 0, 0, 0, 0, 0x5000003D , "XXX", 2, 1

IRDFlash, 15, "Flash 1(28F160)", 0xd0, 0x50000000, 0x200000, 2, 2, 0

IRD, 16, "301-3100 (P17x 2 Flash)", 7, 3, 1, 1, 2, 2, 0x7FFFFF40, 0x7FFFFF44, 0x7FFFBFE0, 0x7FFFFFA8, 0x7FFDFFF4, 0x7FFDFFF8, "10E", 3, 4

IRDFlash, 16, "Flash 1(U10-M29W160)", 0x22C4, 0x7FC00000, 0x200000, 2, 2, 0

IRDFlash, 16, "Flash 2(U12-M29W160)", 0x22C4, 0x7FE00000, 0x200000, 2, 2, 0

IRD, 17, "301-3100 (Single Flash)", 7, 3, 1, 1, 2, 2, 0x7FFFFF40, 0x7FFFFF44, 0x7FFFBFE0, 0x7FFFFFA8, 0x7FFDFFF4, 0x7FFDFFF8, "10E", 3, 4

IRDFlash, 17, "Flash 1(U12-M29W320DT)", 0x22CA, 0x7FC00000, 0x400000, 2, 2, 0

IRD, 18, "508-5800", 6, 3, 1, 1, 2, 2, 0x7FFFFF40, 0x7FFFFF44, 0x7FFFBFE0, 0x7FFFFFA8, 0x7FFDFFF4, 0x7FFDFFF8, "10S", 3, 4 
IRDFlash, 8, "Flash 1(U6-29DL323)", 0x2250, 0x7F800000, 0x400000, 2, 2, 0 
IRDFlash, 8, "Flash 2(U7-29DL323)", 0x2250, 0x7FC00000, 0x400000, 2, 2, 0

// ============================================================================

// ============================================================================

// ============================================================================

// ============================================================================

// Flash Definitions/Flash Sector Definitions

Flash, 1, "29F400BT", 0x2223, 0x80000, 1, 1, 0, 1, 11, 1

Sector, 1, 7, 0x00000, 0x10000

Sector, 1, 1, 0x70000, 0x8000

Sector, 1, 1, 0x78000, 0x2000

Sector, 1, 1, 0x7A000, 0x2000

Sector, 1, 1, 0x7C000, 0x4000

// STMicro M29W320DB - 32MBit

Flash, 2, "M29W320DB", 0x22CB, 0x400000, 1, 1, 0, 1, 67, 1

Sector, 2, 1, 0,       0x4000		// 16 KByte

Sector, 2, 1, 0x4000,  0x2000		// 8  KByte

Sector, 2, 1, 0x6000,  0x2000		// 8  KByte

Sector, 2, 1, 0x8000,  0x8000		// 32 KByte

Sector, 2, 63,0x10000, 0x10000	// 64 KByte 63 sectors

Flash, 3, "29DL323T", 0x2250, 0x400000, 1, 1, 0, 1, 71, 1

Sector, 3, 63,0		,0x10000

Sector, 3, 8, 0x3f0000,0x2000

Flash, 4, "29F400BB", 0x22AB, 0x80000, 1, 1, 0, 1, 11, 1

Sector, 4, 1, 0,       0x4000

Sector, 4, 1, 0x4000,  0x2000

Sector, 4, 1, 0x6000,  0x2000

Sector, 4, 1, 0x8000,  0x8000

Sector, 4, 7, 0x10000, 0x10000

Flash, 5, "29LV160DT", 0x22C4, 0x200000, 1, 1, 0, 1, 35, 1

Sector, 5, 31,0       , 0x10000

Sector, 5, 1, 0x1f0000,  0x8000

Sector, 5, 1, 0x1f8000,  0x2000

Sector, 5, 1, 0x1fa000,  0x2000

Sector, 5, 1, 0x1fc000,  0x4000

Flash, 6, "29LV160DB", 0x2249, 0x200000, 1, 1, 0, 1, 35, 1

Sector, 6, 1, 0      ,  0x4000

Sector, 6, 1, 0x4000 ,  0x2000

Sector, 6, 1, 0x6000 ,  0x2000

Sector, 6, 1, 0x8000 ,  0x8000

Sector, 6, 31,0x10000, 0x10000

Flash, 7, "M29F400T", 0xd5, 0x80000, 1, 1, 0, 1, 11, 1

Sector, 7, 7, 0,       0x10000

Sector, 7, 1, 0x70000,  0x8000

Sector, 7, 1, 0x78000,  0x2000

Sector, 7, 1, 0x7A000,  0x2000

Sector, 7, 1, 0x7C000,  0x4000

// STMicro, M29W800AT, 1 MByte, Top Boot

Flash, 8, "M29W800AT", 0xd7, 0x100000, 1, 1, 0, 1, 19, 1

Sector, 8, 15,0,       0x10000

Sector, 8, 1, 0xF0000,  0x8000

Sector, 8, 1, 0xF8000,  0x2000

Sector, 8, 1, 0xFA000,  0x2000

Sector, 8, 1, 0xFC000,  0x4000

// Intel, 28F320J5, 32MByte, 32-128KByte Blocks

Flash, 9, "28F320J5", 0x14, 0x400000, 1, 1, 0, 2, 32, 0

Sector, 9, 32, 0, 0x20000

// Intel, 28F320J3A, 32MByte, 32-128KByte Blocks

Flash, 10, "28F320J3A", 0x16, 0x400000, 1, 1, 0, 2, 32, 0

Sector, 10, 32, 0, 0x20000

Flash, 11, "28F160", 0xd0, 0x200000, 1, 1, 0, 2, 32, 1

Sector, 11, 32, 0, 0x10000

Flash, 12, "28F800B", 0xd5, 0x100000, 1, 1, 0, 2, 11, 1

Sector, 12, 1, 0,       0x4000

Sector, 12, 1, 0x4000,  0x2000

Sector, 12, 1, 0x6000,  0x2000

Sector, 12, 1, 0x8000, 0x18000

Sector, 12, 7, 0x20000,0x20000

Flash, 13, "28F800B5-B", 0x889d, 0x100000, 1, 1, 0, 2, 11, 0

Sector, 13, 1, 0,       0x4000

Sector, 13, 1, 0x4000,  0x2000

Sector, 13, 1, 0x6000,  0x2000

Sector, 13, 1, 0x8000, 0x18000

Sector, 13, 7, 0x20000,0x20000

Flash, 14, "28F320S3/5", 0xd4, 0x400000, 1, 1, 0, 2, 64, 1

Sector, 14, 63, i * 0x10000, 0x10000

// STMicro M29W320DT - 32MBit

Flash, 15, "M29W320DT", 0x22CA, 0x400000, 1, 1, 0, 1, 67, 1

Sector, 15, 63,0x0,	 0x10000		// 64 KByte 63 sectors

Sector, 15, 1, 0x3F0000, 0x8000		// 32 KByte

Sector, 15, 1, 0x3F8000, 0x2000		// 8  KByte

Sector, 15, 1, 0x3FA000, 0x2000		// 8  KByte

Sector, 15, 1, 0x3FC000, 0x4000		// 16 KByte

// AMD AM29LV320DT - 32 MBit

Flash, 16, AM29LV320DT", 0x22F6, 0x400000, 1, 1, 0, 1, 71, 1

Sector, 16, 63,0x0,	   0x10000		// 64 KByte 63 sectors

Sector, 16, 8, 0x3F0000, 0x2000		// 8  KByte

// ============================================================================

// ============================================================================

// ============================================================================

// ============================================================================

// Flash Manufacturer Codes

FlashMfg, 0x01, "AMD"

FlashMfg, 0x20, "STMicro"

FlashMfg, 0x89, "Intel"

FlashMfg, 0xAD, "Hynix"

FlashMfg, 0xB0, "Intel"

// ============================================================================

// ============================================================================