English

ODSEdit Ver. 1.6

# User Manual

I N D E X

| 1. ODSEdit Introduction ------------------------------- |  | 3 |
| --- | --- | --- |
|  | 1.1 ODSEdit System Configuration & Functions |  |
|  | 1.2 Definition |  |
| 2. Installation --------------------------------------- |  | 4 |
|  | 2.1 ODSEdit Installation |  |
|  | 2.2 Serial Cable Connection |  |
| 3. System Configuration ------------------------------- |  | 4 |
|  | 3.1 Overview |  |
|  | 3.2 COM Port Setting |  |
|  | 3.3 Upload Type Setting |  |
|  | 3.4 SatcoDX Type Setting |  |
|  | 3.5 CSV File Format Setting |  |
| 4. Channel Information Management --------------------- |  | 5 |
|  | 4.1 Overview |  |
|  | 4.2 Channel Information Uploading from STB to PC |  |
|  | 4.3 Channel Information Downloading from PC to STB |  |
|  | 4.4 Channel Information Editing |  |
|  | 4.5 Favorite List Editing |  |
|  | 4.6 Automatic Adding Channel Information by SatcoDX |  |
|  | 4.7 Export channel to CSV File Format |  |
| 5. Satellite Information Management --------------------- |  | 12 |
|  | 5.1 Overview |  |
|  | 5.2 Satellite Information Uploading from STB to PC |  |
|  | 5.3 Satellite Information Downloading from PC to STB |  |
|  | 5.4 Satellite Information Editing |  |
|  | 5.5 Automatic Adding of Satellite Information using SatcoDX |  |
| 6. Main S/W Management ----------------------------- |  | 15 |
|  | 6.1 Overview |  |
|  | 6.2 Main S/W Uploading from STB to PC |  |
|  | 6.3 Main S/W Download downloading from PC to STB |  |

- ODSEdit Introduction

  - ODSEdit System Configuration & Functions

| Applicable STB Model | ODS2000 F, CI, FP, CIP ODS3000 F, CI |
| --- | --- |
| System OS | Windows 98 / ME / 2000 |
| Disk Memory Size | More than 500Kbyte |
| Functions | (Version 1.6) - Support Favorite Channel Edit - Support Channel Move Function - Support PCH(Preset Channel) - Export Channel to CSV(Excel File Format) (Version 1.2) - Support Positioner Model (ODS 2000FP & CIP) (Version 1.0) - Uploading / Downloading Main S/W, Channel & Satellite Information between STB & PC - Editing Channel & Satellite Information - Getting SatcoDX Information via Internet or file |

  - Definition

-. Serial Cable : This is a connection cable between PC to STB. Both ends consists of  RS232C 9 Pin female connector and No.2, 3, 5 pin should be

connected 1:1, not a twisted match.

-. Main S/W : This is the main program to operate STB. Generally

“ Upgrading” means to change this Main S/W to a new version one.

In the PC its file is displayed in * .PGM.

-. Channel Information : This means all receivable satellites channel

program list. In the PC its file displayed in * .CHA.

-. Satellite Information : This means factory pre-setting of all the satellites & transponders information. In the PC its file is displayed * .SAT.

-. PCH : Preset Channel or Preprogrammed Channel. If you execute the Factory Default, this channel is restored. In the PC its file is displayed *.PCH.

-. CSV : This is a file format which separate each field by comma(,). This is for data exchange between different programs. You may edit the CSV file in Microsoft Excel. In the PC its file is displayed *.CSV.

- Installation

  - ODSEdit Installation

- Run Odsedit_xxxx.exe, and the installation program starts. xxxx is version no. and 0160 means version 1.6.

- Follow the installation program instruction in order, and [ODSEdit] group is created in the [Start->Program] in the MS Window. Choose ODSEdit of its group, and the program starts.

  - Connecting Serial Cable

- Connect the prepared Serial Cable (1:1 one, not a twisted one) between STB& PC.

- You should check if PC’s port is COM1 or COM2.

- System Configuration

  - Overview

Run ODSEdit and click [Tool -> Configuration] menu or . Then,

Configuration screen is displayed. You can set up STB connection &

SatcoDX connection type, the setting values are maintained.

  - COM Port Setting

You should choose which PC port, COM1 / COM2, is connected to STB using Serial Cable. Speed & other factors are automatically set.

  - Upload Type Setting

This shows the operation type of uploading from STB.

In case of [Auto], ODSEdit receives a file automatically uploading a file from STB. You should choose [File-> Upload] menu or  in the ODSEdit program uploading a file from STB in case of the [Manual].

When you choose “ Auto”, ODSEdit may make a wrong operation caused by Serial Port problem. For this reason, the recommendable type is the “Manual”.

  - SatcoDX Type Setting

This shows the current connection type to get information from SatcoDX.

When you are not in the Internet connection mode, you choose [File Mode], which uses the downloaded file selecting SatcoDX Homepage’s Receive File.

You are in the Internet connection mode; you should choose [Internet] type, which makes ODSEdit receive the selected satellite TP & Channel

Information connecting to SatcoDX immediately.

  - CSV File Format Setting

This is a format for sending current channel as a CSV file. [Simple Channel List] shows channel number, channel name and Video/Audio/PCR PID only. [Detail Channel List] shows Simple Channel List and information of 위성 each channel in detail.

- Channel Information Management

  - Overview

Channel Information is the channel program list receivable from satellites.

In the STB, you can check it in the [Main Menu->Edit Channel] menu.

You can easily change or edit the Channel Information using ODSEdit in your

PC after downloading it from STB to PC easily. After changing or editing

Channel Information in the PC, you can transmit the Channel Information to

STB again.

  - Uploading Channel Information from STB to PC

- When you enter [Main Menu->Installation->Receiver Upgrade] of STB menu and select [2 Channel], STB turns into transmission stand-by mode.

- If ODSEdit’s Upload Type is Auto, the transmission is started immediately. In case of Manual Type, on the contrary, you should select [File->Upload] menu or tool bar,  , for transmission.

- When the transmission is finished, chaninfo.cha file is created in your PC  and Channel Information is displayed. 
In case of Positioner Model, the following message is shown.

  - Downloading Channel Information from PC to STB

- Plug off STB power cord.

- Select [File->Open] and choose the file that you want to send. Its file is * .CHA.
If you are editing Channel Information file, store it to select [File->Save].

- Select [File->Download] or tool bar, .

- When the following screen is displayed and plug on the STB power cord,  the transmission is started.

- When the transmission is started, the transmission rate is shown on the STB LED display at the same time.

- When the following screen is displayed, Downloading is finished.

At this time STB LED is changed to [INST] mode for installation. In this stage, you don’t need turn off STB.  After Installation is finished, STB LED is changed to [End] and turns into Stand-By mode. 
[End] means the downloading and installation are succeeded. In case  [Fail] is displayed, errors are arisen during the installation. Then, you have to repeat the same procedure and when the result is same, please contact the dealer.

  - Channel Information Editing

    - Editing Screen Configuration

Channel Information editing screen consists of 3 sections like the following

screen , A.Satellite, B.Transponder, C.Channel. 3 sections are operated

relatively.

For example, if you select a satellite of Satellite section, only the information on the selected satellite is displayed in Transponder & Channel sections. In the same manner, if you select a TP of Transponder section, only the channels of the selected TP are displayed.

When you want to see all channels, select [View All Channel].Same as before, if you select a channel, TP and satellite of the selected channel are displayed in shadow.

A. Satellite Section

창

B. Transponder Section

C. Channel Section

    - Adding a new satellite

  - Click the mouse right button in Satellite Section and select [Add].

  - When the satellite name entering screen is displayed, click the right side arrow and select a satellite name among the factory preset satellite names. If the satellite name you want to add is not in the factory preset satellite name list, you can enter its name by yourselves.

  - Select [OK], and the new satellite name is registered.

    - Editing a satellite

  - Double click the satellite you want to edit in Satellite Section or click the mouse right button and select [Edit].

  - When the satellite name entering screen is displayed, you can edit the satellite name.

  - Select [OK], and the satellite name is changed.

    - Deleting a satellite

  - Select the satellite you want to delete in Satellite section and click the mouse right button and select [Delete].

  - Keep in mind that if you delete the satellite, its TP & Channel information are also deleted.

    - Adding a new TP

  - Select the satellite into which you want to add a TP in Satellite section.

  - In Transponder section, click the mouse right button and select [Add].

  - As below, the left section shows factory preset TP list and the right section is for adding the TP.

If you want to use the preset TP, double click the TP and its information is shown in the right section. If you want to enter a new TP information, you can enter its information in the left section directly.

  - After editing, click [OK] to store.

    - Editing a TP

- Double click the TP you want to edit or click the mouse right button and select [Edit].

- After editing, click [OK] to store.

    - Deleting a TP

- Select the TP you want to delete in Transponder section and click the mouse right button and select [Delete].

- Keep in mind that if the TP is deleted, its channel list is also deleted.

    - Adding a new channel

- Select a place for a new channel in Satellite section and Transponder section.

- In Channel section, click the mouse right button and select

[Add].

- Add a new channel information into a channel information screen and

click [OK] to store.

    - Editing a channel

- Double click the channel you want to edit in Channel section or click the mouse right button and select [Edit].

- After editing, click [OK] to store.

    - Deleting a channel

- Click the mouse right button at the channel you want to delete and

select [Delete].

    - Channel Move

- Channel Move needs [View All Channel] option, select [View All Channel] option.

- Click the mouse right button at the Channel Window you want to move and Select [Move To…].

- Select a place for move channel. Click the mouse right button on the place and Select […Move Here].  The Channel will move to the place.

    - Channel Sort

- Channel Sort needs [View All Channel] option, select [View All Channel] option.

- Click the mouse right button at the Channel Window and Select [Sort].

- There are four kinds of sort orders, Channel name[A-Z], Channel name[Z-A], Satellite, Default, you can select one of them.

  - Favorite List Editing

    - Favorite List Overview

- You can see “Favorite List” after select “View Favorite” option & “View All Channel” option.
,

A. Favorite List Window

B. Channel Window

If the Channel window is a TV Channel you can see the “Favorite List” of TV.  If the Channel window is a Radio Channel you can see the “Favorite List” of Ratio.

    - Favorite List – Channel Add

- Select [View All Channels] and [View Favorites] option to make “Favorite List” active.

- Select the name of “Favorite Group” which you want to add a channel.

- Select [Add to favorite] after click the mouse right button at the channel which you want to add in “Favorite” on channel window.  If select [Add to favorite], the channel will be added at “Favorite List” Window.

    - Favorite List – Channel Deleting

- Select [View All Channel] and [View Favorite] option, Favorite List will be active.

- Select the name of “Favorite Group” you want to delete.

- Click the right mouse button on the channel which you want to delete at the Favorite List window and Select [Delete] then the channel will be deleted.

    - Favorite List – Channel Move

- Select [View All Channel] and [View Favorite] option, Favorite List will be active.

- Select the name of “Favorite Group” you want to move.

- Click the right mouse button on the channel which you want to move at the Favorite List window and Select [Move To…].

- Select a place for move channel. Click the mouse right button on the place and Select […Move Here].  The Channel will move to the place.

    - Favorite List – Channel Sort

- Select [View All Channel] and [View Favorite] option, Favorite List will be active.

- Click the right mouse button at the Favorite List window and Select [Sort].

- There are four kinds of sort order, Channel name[A-Z], Channel name[Z-A], Satellite, Default, you can select one of them.

    - Favorite List – Group Change Name

- Select [View All Channel] and [View Favorite] option, Favorite List will be active.

- Click the right mouse button at the Favorite List window and Select [Rename Group].

- Enter the Favorite Name and Click [OK] button.

  - Automatic adding a channel information by SatcoDX

    - In case configuration’s SatcoDX type is file mode

- Click the mouse right button at the satellite into which you want to add a

channel in Satellite section and select [Upgrade by SatcoDX].

- When file selection window is shown, select the satellite file downloaded

from SatcoDX Homepage. The file is *.SDX.

- If SatcoDX satellite file has a new TP and channel information, the

channel information will be changed newly.

    - In case configuration’s SatcoDX type is Internet

- Click the mouse right button at the satellite into which you want to add a channel in Satellite section and select [Upgrade by SatcoDX].

- Then, automatically SatcoDX Server is connected and the selected

satellite information is transferred and TP and channel information is

renewed.

- If there is no [East] information on the selected satellite in Satellite

section, automatic adding by Internet is not supported.

  - Export Channel to CSV file

    - Export current channel to CSV file

- Loading the channel which you want to export to CSV file.

- Select [File->Save As] and Select [Excel File(*.CSV)] as a [File Type], Input a filename, Click [OK].

    - CSV file Edit

- You may edit CSV file with Microsoft Excel.

- Double click the CSV file you made or Run Microsoft Excel and select [File->Open], [text file(*.prn;*.txt;*.csv)] for the [File Type], then the CSV file will be opened.

- Please refer “Micro Excel User Guide” when you edit the CSV file with Microsoft Excel.

- Satellite Information Management

  - Overview

Satellite Information means the factory preset satellite and TP information.

You can see this information in STB [Main Menu ->Installation -> Manual

Channel Search] menu.

By ODSEdit, you can easily change and edit this Satellite Information

downloading its file from STB to PC and resend the file to STB from PC.

  - Uploading Satellite Information from STB to PC

- In STB [Main Menu->Installation->Receiver Upgrade]menu, select 
[3 Satellite] and the STB turns to stand-by mode.

- If ODSEdit’s Upload Type is Auto, the transmission is immediately started. In Manual type, select [File->Upload] menu or tool bar, , and transmission will be started.

- When transmission is finished, satinfo.sat file is available in the PC and

Satellite Information will be displayed.

  - Download Satellite Information from PC to STB

- Plug off STB power cord.

- Select [File->Open] and choose the file that you want to send. The file extension is * .SAT.
If you are editing Satellite Information file, store it to select [File->Save].

- Select [File->Download] or tool bar, .

- When the following screen is displayed and plug on the STB power cord,  the transmission is started.

- When the transmission is started, the transmission rate is shown on the STB LED display at the same time.

- When the following screen is displayed, Downloading is finished. 

At this time STB LED is changed to [INST] mode for installation. In this stage, you don’t need turn off STB.  After Installation is finished, STB LED is changed to [End] and turns into Stand-By mode. 
[End] means the downloading and installation are succeeded. In case  [Fail] is displayed, errors are arisen during the installtion. Then, you have to repeat the same procedure and when the result is same, please contact the dealer.

  - Editing Satellite Information

    - Editing Screen Configuration

Satellite Information editing screen consists of 2 sections like the following screen, A.Satellite, B.Transponder. 2 sections are operated relatively.

For example, if you select a satellite of Satellite section, only the information on the selected satellite is displayed in Transponder.

A. Satellite Section

B. Transponder Section창

    - Adding a new satellite

  - Click the mouse right button in Satellite Section and select [Add].

  - When the satellite information enter screen is displayed, enter a new satellite location into [Location] and a new satellite name into [Satellite Name]. [Location] is satellite degree from the east and it is same as SatcoDX’s satellite number. Its number should be 4 digits number adding “0” in the first place. If the wrong location data is entered, you can’t do automatic satellite information adding by SatcoDX via Internet.

  - Select [OK], and the new satellite is registered.

    - Editing a satellite

- Double click the satellite you want to edit in Satellite Section or click the mouse right button and select [Edit].

- When the satellite name entering screen is displayed, you can edit the satellite name.

- Select [OK], and the satellite name is changed.

    - Deleting a satellite

- Select the satellite you want to delete in Satellite section and click the mouse right button and select [Delete].

- Keep in mind that if you delete the satellite, its TP information are also deleted.

    - Adding a new TP

- Select the satellite into which you want to add a TP in Satellite section.

- In Transponder section, click the mouse right button and select [Add].

- Adding a new TP into TP information entering screen.

- Click [OK] to store.

    - Editing a TP

- Double click the TP you want to edit or click the mouse right button

and select [Edit].

- After editing, click [OK] to store.

    - Deleting a TP

- Click the mouse right button at TP you want to delete in Transponder section and select [Delete].

  - Automatic adding Satellite Information by SatcoDX

    - In case configuration’s SatcoDX type is file mode

- Click the mouse right button at the satellite into which you want to add a TP in Satellite section and select [Upgrade by SatcoDX].

- When file selection window is shown, select the satellite file

downloaded from SatcoDX Homepage. The file is *.SDX.

- If SatcoDX satellite file has a new TP information, the channel

information will be changed newly.

    - In case configuration’s SatcoDX Type is Internet

- Click the mouse right button at the satellite into which you want to add

a TP in Satellite section and select [Upgrade by SatcoDX].

- Then, automatically SatcoDX Server is connected and the selected

satellite information is transferred and TP information is renewed.

- If there is no [East] information on the selected satellite in Satellite

section, automatic adding by Internet is not supported.

## Main S/W Management

  - Overview

This is the function to uploading and downloading STB Main S/W.

This is for upgrading STB main S/W version by downloading a new main S/W , which is generally downloaded from Homepage, from PC to STB.

The following uploading & downloading should be implemented under the condition that STB and PC are connected with RS232C 1:1 Serial Cable and ODSEdit is running in the PC.

  - Uploading a Main S/W from STB to PC

- Select [1 Main S/W] in STB [Main Menu->Installation->Receiver Upgrade] menu and STB turns to stand-by mode.

- If ODSEdit’s Upload Type is Auto, the transmission is started immediately. In Manual, select [File->Upload] menu or tool bar, , the transmission is

started.

- After transmission is finished, mainpg.pgm file is created in the PC.

  - Downloading a Main S/W from PC to STB

- Plug off STB power cord.

- Select the Main S/W you want to transmit in [File->Open] menu. Main S/W

File is *.PGM.

- Select [File->Download] or tool bar, .

- After the following screen is displayed, plug in STB power cord, and the transmission is started.

- When the transmission is started, the transmission rate shown in PC is displayed in STB LED display at the same time.

- When the following screen is displayed, the downloading is finished.

At this time STB LED is changed to [INST] mode for installation. In this stage, you don’t need turn off STB.  After Installation is finished, STB LED is changed to [End] and turns into Stand-By mode. 
[End] means the downloading and installation are succeeded. In case  [Fail] is displayed, errors are arisen during the installation. Then, you have to repeat the same procedure and when the result is same, please contact the dealer.