How to upgrade

|  | PC <-> STB |
| --- | --- |

- Pull out the STB power plug.

- Connect “Computer” and “STB” using RS-232C Serial Cable.

[ ODS2000 model ]

(ex. ODS2000F, ODS2000CI, ODS2000CIV)

| # RS-232C Serial Cable |  |  |  |
| --- | --- | --- | --- |
| Type : 1:1 Cable |  |  |  |
| 2 |  |  | 2 |
| 3 |  |  | 3 |
| 5 |  |  | 5 |
| PC Serial Port |  | STB Serial Port |  |
| (Female, 9Pin) |  | (Female, 9Pin) |  |

[ other models ]
   (ex. ODS3000, ODS4000, C-box, T-box, etc)

| # RS-232C Serial Cable |  |  |  |
| --- | --- | --- | --- |
| Type : Null Modem Cable |  |  |  |
| 2 |  |  | 2 |
| 3 |  |  | 3 |
| 5 |  |  | 5 |
| PC Serial Port |  | STB Serial Port |  |
| (Female, 9Pin) |  | (Female, 9Pin) |  |

- Run “ODSEdit” or “ODSLoader” on your Computer.

- Choose “COM port” from menu “Tool->Configuration”.

- Select “File->Download” from menu after open the file you want to download.

- Transmission will be started after plug on the STB.

- When there is a message “End” on the STB, it means upgrade is successfully finished. If there is a message “Fail” on the STB, it means upgrade is fail.

※ ODSEdit or ODSLoader can download at Download Utility on Opentech Homepage (http://www.opentech.co.kr).